precautions

1. Network & Connectivity

- Same network: ESP32 and device running the website must be on the same Wi-Fi.

- Static IP: Assign a static IP to ESP32 to avoid your website breaking when the router reassigns IPs.

- Router firewall: Make sure local ports (usually 80 or 8080) are open for internal access.

- Wi-Fi stability: Avoid networks with frequent drops; ESP32 can silently disconnect.

2. Security

- No open network exposure: Don’t expose the ESP32 directly to the internet without proper authentication.

- CORS headers: Use Access-Control-Allow-Origin: * for local development only; restrict in production.

- Input sanitization: Validate and sanitize incoming requests to avoid malformed JSON or injection.

- Avoid sensitive data: Don’t send passwords or secrets in plain HTTP; consider HTTPS if needed.

3. Hardware Safety

- Power stability: Use a reliable USB power source; ESP32 may crash with unstable voltage.

- Temperature: Avoid running the ESP32 in hot or enclosed areas.

- Connections: Make sure no short circuits when using additional sensors or peripherals.

4. Server & Code Practices

- Proper headers: Handle OPTIONS for preflight requests; otherwise POST fetch fails.

- Timeout handling: Return responses quickly; browser fetch may timeout.

- Error handling: Always respond with JSON, even on error: { "response": "error message" }.

- Avoid blocking code: Don’t use long delays in the loop; use async patterns if possible.

5. Browser & Website

- Use HTTP for local: Browsers block fetch() to HTTP from HTTPS sites unless configured.

- CORS errors: If fetch fails, check Access-Control-Allow-Origin header and server IP.

- Refresh & retry logic: Implement retry in JS if ESP32 temporarily disconnects.

- Debugging: Keep a console log in the website and serial monitor for ESP32.

6. Testing & Maintenance

- Serial monitor: Monitor ESP32 output for connection issues.

- Check IP conflicts: Ensure no other device has the same static IP.

- Firmware updates: Flash stable code; avoid experimental builds in production.

- Backup: Keep a copy of your working code in case you need to reflash.