1️⃣ STARTING OLLAMA (CORRECT WAY)

✅ Step 1: Open Command Prompt as Administrator

This is important to avoid the “access denied” and port errors you saw earlier.

cmd

Right-click → Run as Administrator

✅ Step 2: Start Ollama server

Ollama automatically starts its HTTP server when you run a model.

ollama serve

🔹 Expected output (example):

Listening on 127.0.0.1:11434

⚠️ Do NOT close this window
This window must stay open while ESP32 is running.

✅ Step 3: (Optional) Pull model if not already installed

You already have models, but for safety:

ollama pull llama3

✅ Step 4: Test Ollama locally (VERY IMPORTANT)

Open another Command Prompt (normal is fine):

curl http://127.0.0.1:11434/api/generate ^

-H "Content-Type: application/json" ^

-d "{\"model\":\"llama3\",\"prompt\":\"hello\"}"

✅ Expected output (streaming JSON):

{"response":"Hello! How can I help you today?"}

If this fails → ESP32 will NEVER work

2️⃣ ALLOW OLLAMA FOR LAN (ESP32 ACCESS)

✅ Step 5: Get your PC’s LAN IP

ipconfig

Example:

IPv4 Address . . . : 192.168.1.5

This is the IP your ESP32 must use.

✅ Step 6: Start Ollama bound to all interfaces

Stop current Ollama server (Ctrl + C), then run:

set OLLAMA_HOST=0.0.0.0:11434

ollama serve

Now Ollama is reachable by ESP32.

✅ Step 7: Test from LAN IP

curl http://192.168.1.5:11434/api/generate ^

-H "Content-Type: application/json" ^

-d "{\"model\":\"llama3\",\"prompt\":\"test\"}"

✅ If this works → ESP32 WILL work.

3️⃣ STOPPING OLLAMA (SAFE WAY)

✅ Method 1: Keyboard (BEST)

In the Ollama terminal:

CTRL + C

❌ DO NOT use Taskkill unless frozen

Only if stuck:

taskkill /IM ollama.exe /F

4️⃣ STARTING THE ESP32 PROJECT

✅ Step 8: Update ESP32 code values

In your ESP32 sketch:

const char* ollamaHost = "192.168.1.5";

const int ollamaPort = 11434;

⚠️ Never use:

localhost

127.0.0.1

ESP32 cannot see your PC’s loopback.

✅ Step 9: Upload ESP32 code

- Board: ESP32 Dev Module

- Port: correct COM

- Baud: 115200

✅ Step 10: Open Serial Monitor

You must see:

Connecting to WiFi...

ESP32 READY

ESP32 IP: 192.168.1.3

5️⃣ TEST ESP32 → OLLAMA DIRECTLY

From your PC:

curl -X POST http://192.168.1.3/ask ^

-H "Content-Type: application/json" ^

-d "{\"question\":\"Who are you?\"}"

✅ Expected:

{"reply":"I am an AI model..."}

6️⃣ STARTING THE WEBSITE

⚠️ IMPORTANT

Opening HTML with file:// WILL FAIL fetch()

✅ Step 11: Start a local web server

From website folder:

python -m http.server 8000

✅ Step 12: Open browser

http://localhost:8000

✅ Step 13: Website config must point to ESP32

fetch("http://192.168.1.3/ask", {

method: "POST",

headers: { "Content-Type": "application/json" },

body: JSON.stringify({ question: userText })

});

7️⃣ STOPPING EVERYTHING (CLEAN SHUTDOWN)

🔴 Stop website server

CTRL + C

🔴 Stop ESP32

- Unplug USB or reset board

🔴 Stop Ollama

CTRL + C

8️⃣ COMMON FAILURE POINTS (FROM YOUR ERRORS)

| Error | Cause |
| --- | --- |
| HTTP Code -1 / -11 | Network timeout / Ollama unreachable |
| ESP32->Ollama fetch failed | Wrong IP / firewall / Ollama not listening on LAN |
| Missing question | Frontend sent wrong JSON key |
| Failed to fetch | file:// or CORS or ESP32 offline |
| PC unreachable | Firewall or wrong IP |
| Address already in use | Ollama already running |

✅ FINAL CHECKLIST (MEMORIZE THIS)

✔ Ollama running
✔ Ollama bound to 0.0.0.0
✔ ESP32 uses PC LAN IP
✔ Website served via HTTP
✔ JSON key = "question"
✔ Same WiFi network

curl http://192.168.1.5/ask ^

-H "Content-Type: application/json" ^

-d "{\"question\":\"hello\"}"